MATERIAL SAFETY DATA SHEET

Date: 18 August 2016

1.IDENTIFICATION OF THE SUBSTANCE/PREPARATION AND THE COMPANY/UNDERTAKING

Product name :         4,4'-Bis(2,3-epoxypropoxy)-3,3',5,5'-tetramethylbiphenyl

Product Code:          TJ-YX4000

Supplier :               GuangZhou TaiJi New Materials CO.,ltd

Address:               602-01, 8 Jin Jin Street, Whampoa District, Guangzhou, China

Emergency telephone :   +86-020-89855706/18620003222

2.COMPOSITION/INFORMATION ON INGREDIENTS

Component Name Content: 4,4'-Bis(2,3-epoxypropoxy)-3,3',5,5'-tetramethylbiphenyl

CAS No.    85954-11-6

Percentage % : 100%

3.HAZARD IDENTIFICATION

Most important hazards: The product consists of a substance of a Category 3 mutagen under EC criteria and a skin sensitiser.

Human health hazards: Dust or small particles may abrade skin and irritate eyes

Classified as a Category 3 mutagen under EC criteria and a skin sensitiser.

Safety hazards: Danger of dust explosion when ground to a small particle size. Electrostatic charges may be generated during handling. Not classified as flammable but will burn.

Environmental hazards:  No specific hazards.

4.FIRST-AID MEASURES

Symptoms and effects: Not expected to give rise to an acute hazard under normal conditions of use.

First Aid - Inhalation:  Remove to fresh air and call a physician

First Aid - Skin:  Immediately wash thoroughly   with soap and water. (Call a physician)

First Aid - Eye:  Flush eye with plenty of water during at least 15min. If persistent irritation occurs, obtain medical attention.

First Aid - Ingestion: Do not induce vomiting. If rapid recovery does not occur, obtain medical attention.

5.FIRE-FIGHTING MEASURES

Specific hazards:  Not classified as flammable but will burn.   Carbon monoxide may be evolved if incomplete combustion occurs.

Extinguishing media-small fires: Dry chemical powder, carbon dioxide, foam, water spray or fog, sand or earth.

Extinguishing media - large fires: Foam, water spray or fog.

Unsuitable extinguishing media:  Water in a jet.

Protective equipment:  Full protective clothing and self-contained breathing apparatus

6. ACCIDENTAL RELEASE MEASURES

Personal precautions:  Avoid raising a dust cloud. Take precautionary measures against static discharge. No specific measures.

Personal protection:  Wear protective clothing specified for normal operations (see Section 8). If risk of inhalation of dust wear general purpose dust respirator (disposable)

Environmental precautions:  No specific measures.

Clean-up methods-small spillage: Shovel up and place in a labelled, sealable container for subsequent safe disposal. Flush contaminated area with plenty of water.

Clean-up methods - large spillage: Transfer to a labelled container for product recovery or safe disposal. Flush away residues with water.

7. HANDLING AND STORAGE

Handling:  Use local exhaust extraction over processing area. Avoid generation or accumulation of dusts. Electrostatic charges may be generated during handling; for example, when crushing, grinding, and sewing or emptying bags, particularly in the preparation of solutions in organic solvents.  Earth all equipment including funnels, chutes anddrums.

Handling temperatures:  Ambient.

Storage:  Keep container tightly closed and dry. Keep away from heat. Keep away from direct sunlight.Palletised loads should be stacked to a maximum of 3 high.Maximum storage time 12 months.

Storage temperatures:  Ambient.

8.EXPOSURE CONTROLS/PERSONAL PROTECTION

Occupational exposure standards: None established

Respiratory protection:  Not normally required. If risk of inhalation of dust wear general purpose dust respirator (disposable)

Hand protection:  PVC gloves.

Eye protection:  Dust-tight monogoggles.

Body protection:  Standard issue work clothes. Safety boots - chemical resistant without lace holes.

9.PHYSICAL AND CHEMICAL PROPERTIES

Physical state:  White or pale yellow powder; granular solids r

Form:  Grain

Colour: White or pale yellow

Odor:    Non irritative odor.

Melting point:  Not determined

Density:  1.149 g/ml

Boiling Point/Boiling Range： 473ºC

Flammability：Flame retardant

Decompositon temperature：NA

Flash point:  > 132ºC

Solubility in water:  0.3mg/kg @25℃

10. STABILITY/REACTIVITY

Stability:  Stable under normal use conditions.

Conditions to avoid: Heat. Strong sunlight for prolonged periods.

Materials to avoid:  Strong acids. Strong oxidising agents.

Hazardous Polymerization: None

Decomposed materials :  Carbon Monoxide and Carbon Dioxide

11. TOXICOLOGICAL INFORMATION

Acute toxicity-oral:  ①LD50 > 5,000 mg/kg    rat, female

3,563mg/kg     rat, male

②LD50 > 1,000 mg/kg   rat

Acute toxicity-dermal:  NA

Eye irritation: Not irritating. But, the dust or small particle may irritate eyes.

Skin irritation:Not irritating. But, the dust or small particle may abrade skin.

Respiratory irritation: Not irritating.

Skin sensitisation:  A Buehler test for sensitisation in the guinea pig was negative, but a Magusson and Kligman maximisation study in the guinea pig was positive and so the substance is provisionally classified as a skin sensitiser.

12. ECOLOGICAL INFORMATION

Mobility:  The substance is likely to distribute between the aquatic and terrestrial environment since it is poorly water soluble and no-volatile.

Degradability: The substance is not subject to aerobic biodegradation. Abiotic degradation in the environment is anticipated as the substance has been shown to have a short half-life.

Accumulation:  The substance is not likely to bioaccumulate in the environment based on the log Pow value of ①2.9 and accumulation value in carp ①<65 and ②<20.

Ecotoxicity:  Practically non-toxic, LC/EC50>100 mg/l, to aquatic organisms (estimated).

TLm48(killfish): ①  3.6mg/L ②  >2.0mg/L

Sewage treatment:  Practically non-toxic, EC50 >100 mg/l, to organisms in sewage treatment plants (estimated).Mutagenicity  Positive both in the Ames test and Chromosomal aberration test. Classified as a Category 3 mutagen under EU criteria.

13. DISPOSAL CONSIDERATIONS

Precautions: See Section 8. Refer to Section 7 before handling the product or containers.

Waste disposal: Recover or recycle if possible. Otherwise incineration, licensed landfill or dispose to licensed disposal contractor.

Product disposal:  Recover or recycle if possible. Otherwise incineration, licensed landfill or dispose to licensed disposal contractor.

Container disposal: Puncture to prevent re-use. Remove all packaging for recovery or waste disposal. Dispose as industrial waste.

14. TRANSPORT INFORMATION

Not dangerous for conveyance under UN, IMO, ADR/RID and IATA/ICAO codes.

Transportation treatment:  Prevent humidity and high temperature.

IATA DGR : Not restricted, as per special provision

IMO: Not restricted.

15. REGULATORY INFORMATION

Applicable Regulations: (Not meant to be all-inclusive-selected regulations represented)

NOTICE:The information herein is presented in good faith and believed to be accurate as of the effective date shown above. However, no warranty, express or implied is given. Requlatory requirements are subject to change and may differ from one location to another; it is the buyer's 
responsibility to ensure that its activities comply with federal, state or provincial, and local laws, The following specific information is made for the purpose of complying with numerous federal, state or provincial, and local laws and requlations. See other sections for fealth and safety information.
US. Toxic Substances Control Act 
All components of this product are on the TSCA Inventory or are exempt from TSCA Inventory 
requirements under 40 CFR 720.30 
European Inventory of Existing Commercial Chemical Substances (EINECS) 
The components of this product are on the EINECS inventory or are exempt from inventory 
requirements.

16. OTHER INFORMATION

Technical contact point:  TaiLuck R&D

Organization ：GuangZhou TaiJi New Materials CO.,ltd

Address/Telephone： 602-01, 8 Jin Jin Street, Whampoa District, Guangzhou, China

+86-020-89855706/18620003222

Person who prepared the GHS:  Manager Name： Yangji 18620003222

DISCLAIMER: This information is based on our current knowledge and is intended to describe the

product for the purposes of health, safety and environmental requirements only. It should not therefore be

construed as guaranteeing any specific property of the product.